Government Center 915 8th Street, Suite 123

Marysville, California 95901-5273

(530) 749-5450 Fax (530) 749-5454

RECREATONAL HEALTH POOL AND SPA SELF-INSPECTION CHECKLIST

This checklist is to help you comply with health department regulations

*It is recommended to post this checklist inside the pool equipment area and complete the checklist monthly

| TURNOVER REQUIREMENTS |  |  |  |  |
| --- | --- | --- | --- | --- |
| POOL GALLONAGE: |  |  |  | SPA GALLONAGE: |
| REQUIRED GPM: |  |  |  | REQUIRED GPM: |
| FENCING/GATES |  |  |  |  |  |  |
| YES |  | NO |  |  |  |  |
| ☐ |  | ☐ | - Fencing in good repair with no gaps exceeding 4 inches and bottom gap not exceeding 2 inches above grade or 4 inches above hardscape (concrete). |  |  |  |
| ☐ |  | ☐ | - All gates are self-closing, self-latching and are kept closed. |  |  |  |
| ☐ |  | ☐ | - No climbable landscaping within 5 ft from top of the fence (measured outside). |  |  |  |
| SAFETY SIGNS/EQUIPMENT |  |  |  |  |  |  |
| ☐ |  | ☐ |  | - All safety signs are posted and are readable: (legible letters/numbers that are not less than 4” in height, unless otherwise required) - Waring- No lifeguard on duty - Diagrammatic illustration of CPR and artificial respiration - Emergency telephone number 9-1-1 Local emergency services, pool name and address - Maximum occupancy - No diving allowed (pools with maximum depth less than 6 feet) - Spa emergency shut-off switch - Spa use warning - No use of pool after dark (pool or/and deck area without lights); - Water is recirculated; do not drink (for spray grounds only) - Keep Closed (on exterior gates/doors) - Emergency Exit sign required on keyless egress when accompanied with keyed gates (after July 1, 1994) - Diarrhea signage (“Person having currently active diarrhea or who have had diarrhea within the previous 14 days shall not be allowed to enter the pool water”). |
| ☐ |  | ☐ |  | - Body hook permanently attached to 12 foot pole is available at pool site. |  |  |
| ☐ |  | ☐ |  | - Life ring with attached rope to span the width of pool available at the pool. Rope in good repair and stored to prevent kinking. |  |  |
| POOL OPERATOR |  |  |  |  |  |  |
| ☐ |  | ☐ |  | - Every public pool shall be under the supervision of a pool operator |  |  |
| POOL CLARITY |  |  |  |  |  |  |
| ☐ |  | ☐ |  | - Water is clear and main drain is clearly visible from pool deck |  |  |
| ☐ |  | ☐ |  | - The pool/spa must not contain any algae or an accumulation of organic debris (leaves, sticks, etc.) |  |  |
| CHEMISTRY/DAILY RECORDS |  |  |  |  |  |  |
| ☐ |  | ☐ |  | - DPD pool test kit is available and in use. Cyanuric acid (Cya) test kit available. |  |  |
| ☐ |  | ☐ |  | - Chemical Concentration - Chlorine: Maintain chlorine levels from - Pools with Cya (2.0-10ppm) - Pools without Cya (1.0-10ppm) - Spas/Wading Pools/Spray grounds (3.0 -10 ppm) |  |  |
| ☐ |  | ☐ |  | - pH: Maintain pH between 7.2 and 7.8 - Cyanuric acid: Maintain cyanuric acid under 100ppm |  |  |
| ☐ |  | ☐ |  | - Daily record log must be kept to record daily chlorine and pH level any day the pool is open (must include weekends and holidays). Cyanuric acid levels must be tested and recorded monthly. - Sperate logs should be kept for each pool/spa. - Log should also be used to record how chemical issues were corrected, backwashing of filters, fecal incidents, or other issues with the pool facility. |  |  |
| SPA REQUIREMENTS |  |  |  |  |  |  |
| ☐ |  | ☐ |  | - The spa temperature shall not exceed 104 °F |  |  |
| ☐ |  | ☐ |  | - An emergency shut-off switch must be available and operational |  |  |
| POOL STRUCTURE |  |  |  |  |  |  |
| ☐ |  | ☐ |  | - Tiles are not cracked, missing or unclean |  |  |
| ☐ |  | ☐ |  | - Depth markers must be present on decking, unbroken and easily readable. For pool built after July 1, 2012 must have depth markers high on the tile line, above the water level. |  |  |
| ☐ |  | ☐ |  | - In pools with a depth of 5 feet or greater there must be a tile line of contrasting color, 4 inches in width across the bottom of the pool at the 4 ½ foot depth. |  |  |
| ☐ |  | ☐ |  | - The pool plaster must be white and free of chips, cracking and pitting |  |  |
| ☐ |  | ☐ |  | - A minimum of 4 feet of unobstructed deck space must be maintained around the pool |  |  |
| ☐ |  | ☐ |  | - All hand and grab rails must be secure |  |  |
| ☐ |  | ☐ |  | - The pool and deck lighting must be operational and maintained. The pool lighting must be wired correctly to an operational GCF to eliminate electrical hazards. |  |  |
| POOL FACILITIES |  |  |  |  |  |  |
| ☐ |  | ☐ |  | - The restrooms, dressing rooms and shower facilities must be maintained with adequate supplies |  |  |
| ☐ |  | ☐ |  | - Sinks and showers must have hot water at 110 °F or below |  |  |
| ☐ |  | ☐ |  | - Must provide an operational drinking fountain |  |  |
| ☐ |  | ☐ |  | - All hose bibs must have anti-siphon valves or vacuum breakers |  |  |
| EQUIPMENT |  |  |  |  |  |  |
| ☐ |  | ☐ |  | - All skimmers are required to be operational, have a skimmer basket, weir and float valve assembly |  |  |
| ☐ |  | ☐ |  | - Main Drain: Main drain must be securely attached and be of VGB/AB1020 compliant |  |  |
| ☐ |  | ☐ |  | - Flow meter must be operational, readable and of correct sizing for the gauge of the plumbing |  |  |
| ☐ |  | ☐ |  | - The must be accurate influent and effluent pressure gauges |  |  |
| ☐ |  | ☐ |  | - An approved automatic chlorinator is required to be maintained and in use at every public pool |  |  |
| ☐ |  | ☐ |  | - No leaking equipment |  |  |
| ☐ |  | ☐ |  | - Pool equipment area must be maintained and free of leaves, plants, garbage, etc. |  |  |
| ☐ |  | ☐ |  | - Filtration equipment must be approved prior to installation and properly maintained. - Cartridge filters must be backwashed or replaced as necessary. - Sand or diatomaceous earth filters must be backwashed as necessary. |  |  |
| ☐ |  | ☐ |  | - The turnover rate of the water must be maintained during hours of pool operation |  |  |
| ☐ |  | ☐ |  | - Any equipment changes must be approved by the Environmental Health Department prior to installation |  |  |
| ☐ |  | ☐ |  | - All pumps, electrical equipment and hand railings must have bonding wire attached and connected to the pool shell rebar |  |  |
| ☐ |  | ☐ |  | - Wastewater Disposal: Wastewater shall be disposed into a sanitary sewer. Wastewater shall not be disposed of unlawfully into storm drains |  |  |