YUBA COUNTY CDSA

915 8th STREET, SUITE 123  MARYSVILLE, CA  95901

(530) 749-5450  FAX (530) 749-5454

WATER QUALITY MONITORING REQUIREMENTS

NON-TRANSIENT, NON-COMMUNITY WATER SYSTEM

| INITIAL TESTING |  |
| --- | --- |
| Constituent |  |
| Primary (Inorganics) | One sample per source |
| Secondary (Aesthetics) | One sample per source |
| Lead & Copper (LCR) | Two sets of samples six months apart (sample sizes based on the number of service connections). |
| Volatile Organic Chemicals (VOCs) | ¼ sampling for 1st year |
| Synthetic Organic Chemicals (SOCs) | One sample per source unless non-vulnerable |
| Radiological – Gross Alpha | ¼ sampling for 1st year |
| Radiological – Radium-228 | ¼ sampling for 1st year |
| Perchlorate | Two samples 5-7 months apart |
| Asbestos | One sample per source unless non-vulnerable |

After initial testing is complete, the following monitoring is required:

| Constituent | Groundwater Source | Standby Source* |
| --- | --- | --- |
| Primary (Inorganics) | Every 3 years Reduced monitoring – Every 9 years | Every 9 years |
| Secondary (Aesthetics) | Once | Once |
| Nitrates | Every year | Every 9 years |
| Nitrites | Every 3 years | Every 9 years |
| LCR | Every 3 years | Every 9 years |
| VOCs | Every 3 years Reduced monitoring – Every 6 years | Every 9 years |
| SOCs | Every 3 years If non-vulnerable - Waived | Every 9 years |
| Gross Alpha | Every 9 years | Every 9 years |
| Radium 228 | Once – 4 consecutive quarters | Once – 4 consecutive quarters |
| Perchlorate | Every 3 years | Every 9 years |
| Asbestos | Every 9 years If non-vulnerable - Waived | Every 9 years |
| Disinfection By-Products (DBPR) | FOR SYSTEMS THAT DISINFECT: One sample per year per treatment plant during the month of warmest water temperature. DBPR testing can be reduced to once every three years if initial sample is <.20 mg/L and .015 mg/L for TTHM & HAA5 respectively. |  |
| Coliform | At least one sample per month shall be collected in accordance with an approved Bacteriological Sample Siting Plan. Systems that disinfect (i.e., chlorinate) shall also collect one sample per quarter at each source. Chlorine residuals must be collected with all coliform samples. |  |

*The California Code of Regulations (Title 22, Chapter 15, Section 64414) requires that a standby source only be used for short-term emergencies of five consecutive days or less, and for less than a total of fifteen calendar days per year.

A state certified laboratory must perform the analyses and, with the exception of lead and copper monitoring, collect the samples for chemical analyses.