YUBA COUNTY CDSA

915 8th STREET, SUITE 123  MARYSVILLE, CA  95901

(530) 749-5450  FAX (530) 749-5454

WATER QUALITY MONITORING REQUIREMENTS

STATE SMALL SYSTEM

| Constituent | Frequency |
| --- | --- |
| Coliform | At least one sample per quarter shall be collected in accordance with an approved Bacteriological Sample Siting Plan. Systems that disinfect (i.e., chlorinate) shall also collect one sample per quarter at each source. Chlorine residuals must be collected with all coliform samples. |

| Constituent | Groundwater Source | Standby Source* |
| --- | --- | --- |
| Primary (Inorganics) | Once | Once |
| Secondary (Aesthetics) | Once | Once |
| Nitrates | Every year | Every 9 years |
| Nitrites | Every 3 years | Every 9 years |
| Nitrate + Nitrite | Every 3 years | Every 9 years |

*The California Code of Regulations (Title 22, Chapter 15, Section 64414) requires that a standby source only be used for short-term emergencies of five consecutive days or less, and for less than a total of fifteen calendar days per year.

A state certified laboratory must perform the analyses and, with the exception of lead and copper monitoring, the samples for chemical analyses must be collected from each source.