YUBA COUNTY CDSA

915 8th STREET, SUITE 123  MARYSVILLE, CA  95901

(530) 749-5450  FAX (530) 749-5454

WATER QUALITY MONITORING REQUIREMENTS

TRANSIENT, NON-COMMUNITY WATER SYSTEM

| Constituent | Frequency |
| --- | --- |
| Coliform | At least one sample per quarter shall be collected in accordance with an approved Bacteriological Sample Siting Plan. TNCs must sample monthly during any month in which 1,000 or more persons are served. Systems that disinfect (i.e., chlorinate) shall also collect one sample per quarter at each source. Chlorine residuals must be collected with all coliform samples. |

| Constituent | Groundwater Source | Standby Source* |
| --- | --- | --- |
| Primary – Inorganics Aluminum Antimony Arsenic Barium Beryllium Cadmium Chromium Fluoride Mercury Nickel Selenium Thallium | Once | Once |
| Secondary Standards Bicarbonate, Carbonate & Hydroxide Alkalinity Calcium Iron Magnesium Manganese pH Specific Conductance Sodium Total Hardness | Once | Once |
| Nitrates | Every year | Every 9 years |
| Nitrites | Every 3 years | Every 9 years |
| Nitrate + Nitrite | Every 3 years | Every 9 years |

*The California Code of Regulations (Title 22, Chapter 15, Section 64414) requires that a standby source only be used for short-term emergencies of five consecutive days or less, and for less than a total of fifteen calendar days per year.

A state certified laboratory must perform the analyses and, with the exception of lead and copper monitoring, collect the samples for chemical analyses.